KB-SELEC

SPECIFICATION SHEET

===================

SWITCH MODE POWER SUPPLY

MODEL 		: KPC9-1-024-0500

INPUT

* I/P  				             : 230V AC, 1-Ph 50Hz

* Range                                                : 180-270V AC

* Fuse Protection                                  : Yes

* I/P MOV                                            : Yes

* Soft Start                                            : Yes

OUTPUT

* Voltage                                               : 24V

* Voltage Range                                     : 24-30V

* Rated Current 	      		   : 50A

* Line & Load Regulation		   : +/- 1%

* Ripple Max	       			   : +/- 1% (P-P)

* Protections

: a) Over Load Protection auto recovery type

b) Short Circuit protection auto recovery type

c) Over Voltage Protection Latching type

d) Over Temperature auto recovery type :80ºC ± 5 deg

* Current Limit                                        : yes

ENVIRONMENTAL

* Operating temperature 0 to 500 C.

* Humidity                                               : < 95% Rh

* Enclosure IP 20 Protection

GENERAL

* Built in Power Factor Correction- 0.99

Isolation                                                : Input to Output – 2KV

Input to Earth – 1.5KV

Output to Earth- 500V

* Potential Free Contact – Alarm contact for output fail

INDICATIONS LED 		                 : a) Input OK

b) Output OK

c) Input Under or Over Voltage(U/O)

d) Output Over Voltage(OVP)

e) Output Over Load(O/L)

f) Over Temp(O/T)

MECHANICAL

* Dimension                                                  : approx. 295W X 350D X 100H mm

* Cooling type                                               : Fan cooled

* Weight                                                        : approx. 5.5 Kgs.