KB-SELEC

SPECIFICATION SHEET

===================

SWITCH MODE POWER SUPPLY

MODEL 		: KPC9-1-110-0100

INPUT

* I/P  				            : 230V AC 1-Ph 50Hz

* Range                                               : 180-270V AC

* Fuse Protection.                               : Yes

* I/P MOV                                          : Yes

* Soft Start                                         : Yes

OUTPUT

* Voltage         		            : 110V DC

* Voltage Range                                 : 100-150V

* Rated Current 	      		: 10A

* Line & Load Regulation		: +/- 1%

* Ripple Max 	       		            : +/- 1% (P-P)

* Protections

: a) Overload Protection auto recovery type

b) Short circuit protection auto recovery type

c) Over Voltage Protection Latching type

d) Over temperature auto recovery type:80ºC ± 5 degrees

* Current Limit.                                 : Yes

ENVIRONMENTAL

* Operating temperature 0 to 500 C.

* Humidity                               : < 95% Rh

* Enclosure IP 20 Protection

GENERAL

* Efficiency                                        : typ 85%

* Built in Power Factor Correction- 0.99

* Isolation                                          : Input to Output – 1.5KV

Input to Earth – 1.5KV

Output to Earth- 500V

* Potential Free Contact – Alarm contact for output fail

INDICATIONS LED 		: a) Input OK

b) Output OK

c) Input Under or Over Voltage (U/O)

d) Output Over Voltage (OVP)

e) Output Over Load (O/L)

f) Over Temp (O/T)

MECHANICAL

* Dimension                                   : approx. 295W X 350D X 100H mm

* Cooling type                                : Fan cooled

* Weight                                         : approx. 6 Kgs